CNC ROUTER MACHINE PROFILE

Machine: Forest Scientific 4×8 CNC Router
Controller Software: Velocity CNC 4.5.24
Motion Hardware: Signal Generator 501A (USB)
Units: Inches

1. Controller

| Parameter | Value |
| --- | --- |
| Software | Velocity CNC |
| Version | 4.5.24 |
| Signal Generator | 501A (USB) |
| Communication | USB High-Speed |
| Lookahead Buffer | 9999 moves |

Notes

- Motion timing generated by hardware controller.

- Large lookahead buffer improves toolpath smoothing and contouring.

2. Motion System

Axis Configuration

| Axis | Drive System | Motor Direction | Motor Type |
| --- | --- | --- | --- |
| X | Rack & Pinion | Negative | Stepper |
| Y | Rack & Pinion | Negative | Stepper |
| Z | Lead Screw | Negative | Stepper |
| A | Rotary | Positive | Stepper |

3. Motion Resolution

X / Y Axes

Drive system: Rack & pinion

| Parameter | Value |
| --- | --- |
| Motor resolution | 200 steps/rev |
| Microstepping | 10 |
| Pinion teeth | 24 |
| Rack pitch | 0.039248 in/tooth |

Resulting Resolution

| Metric | Value |
| --- | --- |
| Position resolution | 0.0004708 in |
| Steps per inch | ~2124 |

Z Axis

Drive system: Lead screw

| Parameter | Value |
| --- | --- |
| Thread count | 5.08 rev/in |
| Position resolution | 0.0000984 in |

Z-axis resolution is approximately 5× finer than X/Y.

4. Stepper Signal Timing

| Parameter | Value |
| --- | --- |
| Step pulse width | 10 µs |
| Minimum time between steps | 8 µs |
| Direction setup | 8 µs |
| Minimum step-direction lag | 8 µs |

5. Feedrate Limits

| Axis | Start Feedrate | Maximum Feedrate |
| --- | --- | --- |
| X | 50 in/min | 400 in/min |
| Y | 50 in/min | 400 in/min |
| Z | 20 in/min | 100 in/min |

6. Acceleration

| Axis | Acceleration | Ramp Rate |
| --- | --- | --- |
| X | 12 in/sec² | 20 in/sec² |
| Y | 12 in/sec² | 20 in/sec² |
| Z | 12 in/sec² | 5 in/sec² |

7. Interpolation / Path Handling

| Parameter | Value |
| --- | --- |
| Maximum Arc Feedrate | 200 in/min |
| Point Feedrate (Linear) | 250 in/min |
| Continuous Contouring Feed Tolerance | 100 in/min |
| Helical Interpolation Accuracy | 0.001 in |
| Arc Radius Tolerance | 0.01 in |

8. Spindle System

Spindle controlled through analog voltage output.

| Parameter | Value |
| --- | --- |
| Control type | Analog (0–10V) |
| RPM Range | 0 – 18,000 rpm |
| Voltage Range | 0 – 10 V |

Conversion

| Voltage | RPM |
| --- | --- |
| 1 V | ~1800 rpm |
| 10 V | 18,000 rpm |

Example:

| Command | RPM | Voltage |
| --- | --- | --- |
| S6000 | 6000 rpm | ~3.3 V |
| S12000 | 12000 rpm | ~6.6 V |
| S18000 | 18000 rpm | 10 V |

9. Spindle M-Code Control

| M Code | Function |
| --- | --- |
| M3 | Spindle On |
| M5 | Spindle Off |

Program cycle:

| Stage | Command |
| --- | --- |
| Program Start | M3 |
| Program End | M5 |

10. Limit and Safety Inputs

| Input | Function |
| --- | --- |
| X- | X Home / Limit |
| Y+ | Y Home / Limit |
| Z+ | Z Home / Limit |
| T0 | Tool Change |
| Front Panel | Emergency Stop |
| Drive Fault | Cycle E-Stop |

All inputs configured Normally Closed (N.C.).

11. Tool Length Sensing

Enabled.

| Parameter | Value |
| --- | --- |
| Probe feedrate | 3 in/min |
| Probe direction | Negative Z |
| Maximum probe travel | 1 in |
| Retract distance | 0.25 in |

Tool length formula

#Machine.Z

12. Program Zero Sensing

Enabled for:

- X axis

- Y axis

- Z axis

| Parameter | Value |
| --- | --- |
| Probe feedrate | 3 in/min |
| Maximum move distance | 3 in |
| Retract distance | 0.25 in |
| Sensor offset | 0.5012 in |

13. Jogging

Jog Rates

| Axis | Slow | Medium | Fast |
| --- | --- | --- | --- |
| X | 5 in/min | 60 in/min | 500 in/min |
| Y | 5 in/min | 60 in/min | 500 in/min |
| Z | 5 in/min | 10 in/min | 100 in/min |

Initial jog rate: Fast

14. G-Code Settings

| Parameter | Value |
| --- | --- |
| Coordinate systems | G54 – G59 |
| Dwell units | Milliseconds |
| Helical interpolation accuracy | 0.001 in |
| Arc radius tolerance | 0.01 in |

15. Default Import Feedrates

| Move Type | Feedrate |
| --- | --- |
| XY Feed | 75 in/min |
| Plunge Feed | 25 in/min |

16. Threading Settings

| Parameter | Value |
| --- | --- |
| Spindle encoder resolution | 1000 pulses/rev |
| Index channel | Disabled |

17. Machine Performance Summary

| Parameter | Value |
| --- | --- |
| XY Position Resolution | 0.0004708 in |
| Z Position Resolution | 0.0000984 in |
| Maximum Programmed Feed | 400 in/min |
| Acceleration | 12 in/sec² |
| Spindle Speed Range | 0 – 18,000 rpm |
| Spindle Control | 0–10V analog |

18. Mechanical System Summary

| Component | Type |
| --- | --- |
| XY Drive | Rack & Pinion |
| Z Drive | Lead Screw |
| Motors | Stepper |
| Spindle Control | VFD via analog voltage |

SPINDLE AND VFD SYSTEM

Spindle

Manufacturer: HSD Mechatronics
Model: AT/MT 1090
Type: Air-cooled CNC spindle motor

| Parameter | Value |
| --- | --- |
| Power | 4.5 kW (≈6 hp) |
| Voltage | 220 V |
| Phase | 3-phase |
| Max Frequency | 300 Hz |
| Maximum Speed | 18,000 rpm |
| Cooling | Air cooled |
| Motor Type | Asynchronous induction |
| Protection Rating | IP50 |

Frequency to RPM Relationship

Spindle speed is proportional to the output frequency from the VFD.

| Frequency | Spindle Speed |
| --- | --- |
| 50 Hz | 3,000 rpm |
| 100 Hz | 6,000 rpm |
| 150 Hz | 9,000 rpm |
| 200 Hz | 12,000 rpm |
| 250 Hz | 15,000 rpm |
| 300 Hz | 18,000 rpm |

Variable Frequency Drive (VFD)

Manufacturer: AutomationDirect
Series: GS2 AC Drive

| Parameter | Value |
| --- | --- |
| Model Series | GS2 |
| Rated Power | 5 hp |
| Input Voltage | 230 V |
| Input Phase | 3-phase |
| Output Type | Variable frequency |
| Motor Control | Sensorless vector / VFD |

Purpose of the VFD:

- Converts fixed AC line power into variable frequency output

- Controls spindle speed

- Provides spindle acceleration and deceleration

- Protects spindle motor from overload conditions

Spindle Control Method

The spindle is controlled directly by the CNC controller using analog voltage control.

Control type:

0–10 V analog signal

Voltage mapping configured in Velocity CNC:

| Voltage | Spindle Speed |
| --- | --- |
| 0 V | 0 rpm |
| 10 V | 18,000 rpm |

Conversion relationship:

1 V ≈ 1,800 rpm

CNC Spindle Commands

Spindle control is issued through standard G-code commands.

| Command | Function |
| --- | --- |
| M3 | Spindle ON (clockwise) |
| M5 | Spindle OFF |
| Sxxxx | Set spindle speed |

Example:

M3 S12000

Result:

- Controller outputs ~6.6 V to the VFD

- VFD produces ~200 Hz output

- Spindle runs at approximately 12,000 rpm

Spindle Startup Sequence

Typical program start sequence:

M3 Sxxxx

Steps executed by the system:

- CNC controller sends M3 command

- Output line activates spindle relay

- Analog voltage output is set according to S command

- VFD ramps motor to target frequency

- Spindle reaches commanded RPM

Spindle Shutdown

M5

Sequence:

- CNC controller disables spindle output

- Analog command voltage drops to zero

- VFD decelerates spindle according to programmed ramp time

Typical Router Spindle Speeds

| Tool Diameter | Typical RPM Range |
| --- | --- |
| 1/8" | 18,000 rpm |
| 1/4" | 14,000 – 18,000 rpm |
| 3/8" | 12,000 – 16,000 rpm |
| 1/2" | 10,000 – 14,000 rpm |

Actual speed depends on:

- material

- tool geometry

- chipload

- machine rigidity

System Architecture

The spindle control chain is structured as follows:

Velocity CNC Controller

│

│ 0–10V analog signal

▼

AutomationDirect GS2 VFD

│

│ variable frequency output (0–300 Hz)

▼

HSD AT/MT 1090 Spindle

Key Operational Limits

| Parameter | Limit |
| --- | --- |
| Maximum spindle speed | 18,000 rpm |
| Maximum VFD frequency | 300 Hz |
| Analog command range | 0–10 V |